Third-Order Fractional-Step Band-Pass Filters

Dražen Jurišić

University of Zagreb

Faculty of Electrical Engineering and Computing

Unska 3, 10000 Zagreb, Croatia

drazen.jurisic@fer.hr

Budimir Lutovac

University of Montenegro

Faculty of Electrical Engineering

G. Washington str., 81000 Podgorica, Montenegro

budo@ac.me

Abstract—In this paper a third-order band-pass (BP) filter with geometrically-symmetrical characteristic is presented, for the first time to the authors knowledge. This filter characteristic is only possible to be obtained by fractional-order (FO) filter design. The desired characteristic was Chebyshev 0.1dB pass-band ripple and with normalized bandwidth B=0.1. Starting from common fourth-order BP transfer function the design was performed numerically using the nonlinear trust-region-reflective, least square algorithm in Matlab. The resulting third-order BP filter approximates very well in the pass-band frequency region, while possessing fractional step in the stop band. The wanted BP transfer function is realized as cascade in two ways: biquads with integer order (IO) and biquads with FO. The obtained design parameters in the IO and FO realizations are compared. Approximation of the constant phase element by Foster I passive RC network is used in a Pspice analysis. That Pspice analysis in a frequency and a time domain are confirming the realizability and stability of the proposed filter, respectively.

Keywords-active filters; analog circuits; circuit synthesis; fractional-order biquads; filtering theory; band-pass filters cascade.

# Introduction

A new emerging interdisciplinary research area has started by application of fractional-order (FO) calculus to generalize more common integer-order (IO) calculus [1]. FO calculus was first mentioned in late 1695 by L’Hospital question to Leibnitz about the 1/2-order derivative, and till today rapid development of the field has emerged because most phenomena in nature can be more accurately described using FO models. In 1974 first book was published [2]. During the last twenty years fractional calculus has gained a lot of attention in many fields of science including mechanics, agriculture, economy, nanotechnology, electrical engineering, and so on. Many applications in electrical engineering are found in circuit theory [3], automatic control [4], electrical transmission lines [5], memristors [6], analog signal processing [7], electrical filters [8], bioengineering [9], and many others.

Continuous-time band-pass (BP) filters play an important role in modern-day systems. This paper applies FO concepts to the design of active-RC filters, having the BP characteristics. It is well known that the BP filters are generally having even order, such as second, fourth, sixth, etc. In this paper we design an odd-order BP filter, which is geometrically symmetrical, and is of third order. To the authors knowledge this is the first time a third order, geometrically symmetrical BP filter is presented. It is expected that this filter should be designed using FO as the only possibility. Although having a lower (third) order it is expected that the proposed BP filter will satisfy the same specifications in the pass band as its fourth-order counterpart. Also the FO filter demonstrate the fractional step stop-band roll-off needed in biomedical applications [9].

The paper is organized as follows. In the second Section basic definitions in FO calculus are given. In the third Section fourth-order IO BP filter is briefly repeated. In fourth Section FO BP filter is designed numerically starting from IO filter. In the fifth Section the presented concepts have been verified by PSpice simulations, showing an excellent agreement.

# Definitions of Fractional-Order Calculus

In definition of fractional derivative, one of the approaches considers fractional differentiations and integrations as limits of finite differentiations and integrations. Its representative is Gruenwald-Letnikov derivative, defined by:

	1

which defines the -th order derivative. There 0<<1 is a non-integer number, and j is an integer number. It possesses a Laplace transform defined by

	2

where the second term represents a sum of initial conditions. When the initial conditions are zero, this becomes

	3

This work was fully supported by the Croatian Science Foundation under project (IP-2016-06-1307) "Fractional analog and mixed systems for signal processing".

simplifying substantially analysis in the frequency domain. In the sequel we will use exclusively the Laplace transformation in the analysis of fractional-order systems.

# Fourth-Order BP Filter Realization

## Cascaded biquads

In this section we will briefly repeat a common procedure of realizing a fourth-order IO BP filter transfer function, and present a corresponding active-RC filter with operational amps.

Two biquads, denoted by “section” 1 and 2, in cascade realize the fourth-order BP filter with the transfer function:

4

where (4) is obtained from the low-pass (LP) prototype filter by the common LP-BP transformation s(s2+2p0)/(Bs) (where B is the bandwidth and p0 is the center frequency), containing the multiplicands with equal pole Q-factors

	5

and pole frequency pairs that are related by a factor A>0 and defined by

	6

The quantity A=(p1/p2)1/2 presents the pole frequency ratio. Conditions (5) and (6) comply with Geffe's algorithm for BP filters [10], which are referred to as geometrically symmetrical filters. Note also that p0 is a geometrical mean value of the pole frequencies p1 and p2 of the two biquads.

To realize (4) two biquadratic sections as shown in Fig. 1, also known as Tow-Thomas circuit have to be connected in cascade. If the circuit contains two fractional capacitances: C, (replaces capacitor C1) and C (replaces capacitor C2), then the transfer function of the circuit is given by:

	7

In the case of =1 and =1, we can compare (7) to the band-pass transfer function of the second-order in general form as in (4), and choosing R5=R6 we have the parameters:

- Tow-Thomas biquad circuit.

	8

If we choose C1=C2=C, R2=R3=R, we have a simple BP filter design procedure:

i) Choose C,

ii) Calculate element values from kBP, p and qp using:

9

## Example

As an example, we consider a fourth-order Chebyshev BP filter with Amax=0.1dB, normalized bandwidth B=0.1, center frequency p0=1 and the corresponding parameters A=1.07156, qp0=8.45057, kBP1=1, and kBP2=2.339535. From (5) and (6) we have p1, p2, qp1, qp2 of the desired transfer function (4), which has the form:

	(10a)

	(10b)

	(10c)

	(10d)

Normalized element values, for two sections as in Fig. 1, denoted by 1 and 2, are calculated from (10) using (9). We choose C=1, and with normalized p0 and B obtain elements in the first two rows of Table I. To calculate denormalized elements with the central frequency f0=1kHz (f0 defines denormalization frequency 0=2f0) we choose C0=1nF, and by that the denormalization impedance R0=(0C0)1= 159.155k. The denormalized element values follow from:

	11

where subscript n means normalized. We calculate elements denormalized to 1kHz using (11) given in the second two rows of Table I. We choose arbitrarily R5=R6=10k.

- Elements of fourth-Order BP filter in Fig. 1

| Section |  | C1=C2 | R1 | R2=R3 | R4 |
| --- | --- | --- | --- | --- | --- |
| Normalized | 1 | 1 | 9.05528 | 1.07156 | 9.05528 |
|  | 2 | 1 | 7.88652 | 0.933221 | 3.37086 |
| 1kHz | 1 | 1.0nF | 1441.2k | 170.54k | 1441.2k |
|  | 2 | 1.0nF | 1255.1k | 148.53k | 536.5k |

# Third-Order BP Filter Realization

In this Section we will derive a new numerical procedure to realize a third-order FO BP filter transfer function. Corresponding active-RC filter follows in Section 5.

## FO Tow-Thomas biquad

If we introduce two constant-phase elements (CPEs) C and C into Tow-Thomas biquad shown in Fig. 1, the transfer function (7) results. It represents a normalized transfer function of the FO BP filter of an order + having a general form

	12

The transfer function (12), has an independent fractional step of 20 dB/dec and 20 dB/dec for frequencies lower and higher, respectively, than the center frequency. This transfer function is capable realizing asymmetric amplitude-frequency characteristic with d.c. and high frequency gains equal to zero.

To obtain amplitude- and phase-frequency characteristic of the transfer function (12), we have to introduce s=j into s, and we have:

	13

By introducing (13) into (12), after some calculations we have:

	14

15

Maximum frequency max of the filter follows from:

16

The two 3dB cut-off frequencies 1 and 2 follow from:

	17

Knowing maximum frequency max, and cut-off frequencies 1 and 2 it is possible to calculate equivalent pole Q-factor of the fractional + order BP filter, which is given by:

	18

The above equations (see also [11]) for the parameters of FO filters are more complicated, than those for IO filters.

## FO 2(1+)-order transfer function realization

The third-order FO transfer function is possible by combining two transfer function as in (12) into cascade. In the first term we set =1, =0.5, while in the second term we can choose =0.5, =1. The obtained transfer function is given by:

19

The six values k1 to k3 for both sections 1 and 2 in (19) are real constants to be obtained numerically to approximate the desired passband amplitude-frequency characteristic.

We apply the nonlinear least square routine from Matlab called lsqcurvefit in the normalized frequency range  from 0.5 to 2. The function to be optimized is the product of two transfer functions (14) using (, ) to be (1, 0.5) and (0.5, 1), i.e. the FO transfer function of the order 21.5=3, and the goal function is the absolute value of (10) (see also [12]). The routine uses the trust-region-reflective algorithm with termination tolerances of the function value and the solution set in Matlab by default to 10−6. The routine lsqcurvefit is defined in Matlab as:

20

where k is the total number of frequency points for the comparison, and x is the filter coefficients (k1_1 to k3_2) vector.

The process of numerical optimization, using (20) in 50 frequency points and fitting the curve in, is shown in Fig. 2.

The obtained normalized 3rd-order FO BP transfer function is Chebyshev with 0.1dB ripple, with p0=1 and B=0.1, and has coefficient values in Table II. The obtained normalized 3rd-order FO BP transfer function (19) has the form:

	(21a)

	(21b)

	(21c)

- Optimization by nonlinear least square routine lsqcurvefit in Matlab.

	(21d)

Using (4)(6) for IO and (14)(18) for FO filters the characteristic values, such as cut-off frequencies 1 and 2 at 3dB and the corresponding Qp and max for two filters are calculated in Table III. From those values it can be seen two cut-off frequencies in FO case are not geometrical mean of the center (maximum) frequency, as it is in IO circuits. Thus, the geometrical symmetry for the separate FO biquads does not hold, whereas for the overall FO filter it is satisfied.

Amplitude-frequency characteristics in dB of normalized transfer functions (10b), (10c), as well as (21b) and (21c) of separate building blocks inside cascade are plot in Fig. 3(a), whereas characteristics of final filters (10d) and (21d) are plot in Fig. 3(b). Note the FO step in the stop-band region.

Stability issues are discussed in [12]. For the system to be stable the angles of all poles in (19) must be greater than the minimum required angle . It is obvious from Table II that the presented filter is stable. Position of poles of (21d) in W-plane is shown in Fig. 4. Regardless of having negative poles FO filter is stable.

- Coefficient values of 3rd-FO Chebyshev 0.1 BP filter

| Section | k1 | k2 | k3 |  |
| --- | --- | --- | --- | --- |
| 1 | 0.086557 | 0.8237 | 1.2372 | 46.684 |
| 2 | 0.168963 | 0.6657 | 0.8085 | 46.687 |

- Parameters of all BP filters at 3dB

| Section |  | 1 | 2 | Qp | max |  |
| --- | --- | --- | --- | --- | --- | --- |
| IO | 1 | 0.87964 | 0.9901 | 8.45057 | 0.93322 | 0.93322 |
|  | 2 | 1.01003 | 1.1368 | 8.45057 | 1.07156 | 1.07156 |
| FO | 1 | 0.878854 | 0.9884 | 8.51047 | 0.93270 | 0.932042 |
|  | 2 | 1.01173 | 1.1381 | 8.48271 | 1.07232 | 1.07308 |

(a)

(b)

- Normalized IO (solid) and FO (dashed) magnitude characteristics of BP filters in Matlab. (a) Building biquads in cascade. (b) Final circuits.

- Pole-zero map in W-plane of the poles of (21d) in Matlab.

# Pspice Simulation and Verification

Consider the multi-amplifier active-RC circuit in Fig. 5, which is capable to physically realize the transfer function of the fractional band-pass filter of order + defined by (12). Note that the circuit is rearranged, when compared to the original circuit in Fig. 1, in order to be able to realize negative coefficients (k2 is negative) using all positive passive components.

To design filter in Fig. 5 we use coefficients in Table II, or the values of the coefficients in (21). We calculate normalized element values by choosing 0=1, C=1, and then other elements follow from k1, k2 and k3 using:

22

We obtain normalized components in the first two rows of Table IV. We choose R5=R6=1. Note also that two capacitors C1 (in section 1) and C2 (in section 2) in Table IV are indeed the fractional capacitors (i.e. where they are needed) having the equivalent value of the impedance as the normal capacitors.

The denormalization in fractional-order circuits is done using the following equations:

	23

where the denormalization frequency is defined as 0 and the denormalization impedance is defined as R0. They differ from (11) in having fractional-order  in the power of 0 (see also [11]). We denormalize the fractional-order transfer function to the center frequency of f0=1kHz, (defining the denormalization frequency to be 0=2f0). If we choose capacitances value to be C0=1nF then we have R0=159.155k, as in the integer-order case. By denormalizing the components in the first two rows we obtain the values of the components in the last two rows in Table IV. We choose arbitrarily R5=R6=10k.

- Fractional-order modified Tow-Thomas circuit.

- Elements of third-Order BP filter in Fig. 5

| Section |  | C1=C2 | R1 | R2 | R3 | R4 |
| --- | --- | --- | --- | --- | --- | --- |
| Norm. | 1 | 1 | 1.21403 | 1 | 0.80828 | 1.553 |
|  | 2 | 1 | 1.50218 | 1 | 1.23686 | 5.91845 |
| 1kHz | 1 | 1.0nF | 193.2k | 159.2k | 128.6k | 1.84M |
|  | 2 | 1.0nF | 239.1k | 159.2k | 196.9k | 941.9k |

## Realization of a constant-phase element (CPE)

In recent times a lot research on physical realization of fractional components is done. However, in the lack of physical components, for Pspice simulation we will use a passive network simulation of them.

The impedance of a fractional capacitance, which is also known as a constant-phase element (CPE), is defined by:

	24

where C is the pseudo-capacitance with units F s1 and 0<α<1 is the order.

Only a fixed approximation for a specific pseudo-capacitance with value of  valid over a bandwidth (around specified center) with acceptable magnitude and phase error is possible. Known methods for approximating the variable s are Carlson’s method, Matsuda’s method, Oustaloup’s method and the continued fraction expansion (CFE) method [13][15], that form a rational function of “s” as a driving point function. Among them, the most efficient is the CFE approximation method, and will be used in this paper.

A fourth-order CFE method approximates the impedance of the fractional capacitance ZCPE(s) in (24). The obtained network is shown in Fig. 6, which is also known as Foster I one-port. Design algorithm, i.e. Matlab procedure from [15] is used.

Passive element values for approximating a CPE with C=1nF at 1 kHz (i.e. Z=159.15k), =0.5, using the Foster I topology in Fig. 6, are given by: Ra=40.063k, Ca=526.27pF, Rb=59.906k, Cb=1.8706nF, Rc=142.89k, Cc=3.3414nF, Rd=1.1718 M, Cd=4.3683nF, Re=17.684 k. The frequency dependence of the absolute value of the impedance Z(j) in dB and the phase angle ()=arg[Z(j)] of the CPE are shown in Fig. 7. In the frequency region from 50Hz to 20kHz the error in constant phase (which must be ideally 45) is less than 2. From 200Hz to 5kHz the error is less than 0.2.

- Constant-phase element simulation by Foster I realisation.

- Impedance- and phase-frequency characteristics of CPE in Fig. 6.

- Amplitude- and Phase-Frequency characteristics of realized filters.

- Transient analysis in Pspice of 3rd-order FO BP realized filter.

For the realization of IO fourth-order BP filter the Tow-Thomas section shown in Fig. 1 is used with computed elements in Table I. For the realization of FO third-order BP filter modified Tow-Thomas section in Fig. 5 is used with computed elements in Table IV, and with two CPEs of order =0.5. A-F and F-F characteristics of the two filters are shown in Fig. 8. Note that in the case of 4th-order filter, the roll-off at low and high frequencies (i.e. in the stopband) is 40dB/decade, respectively, whereas for the 3rd-order filter, with fractional-step attenuation, the roll-off is 30dB/decade. Both filters satisfy the same specifications. The phase characteristics are 180deg in IO and 135deg in FO case, as expected. The step response was also investigated to confirm the third-order FO BP filter stability and is shown in Fig. 9.

# Conclusion

In this paper we present the design of two versions of geometrically symmetrical BP filters (of third and fourth order). To the best authors knowledge this is the first time that the BP filter of third order, which is geometrically symmetrical, is presented. To design latter, which is indeed a fractional-order filter, we have used the least square optimization routine in Matlab. Using FO BP filters, we can fine-tune fractional step in the stop band, because it is possible to obtain more freedom degrees in the design. We have presented the FO BP filter design, which has confirmed that the FO BP filter of third order is feasible and stable. In the future research an analytical design of the fractional step BP filters will be investigated, as well as the extension of the presented principles to the digital domain.

##### Acknowledgment

The paper is funded by Bilateral Project CRO-MONT Authors are thankful for support.

##### References

A. S. Elwakil, “Fractional-order circuits and systems: an emerging interdisciplinary research area,” IEEE Circuits and Systems Magazine, vol. 10, pp. 40–50, November 2010.

K. B. Oldham, and J. Spanier, Fractional calculus: theory and applications, differentiation and integration to arbitrary order. Academic Press, New York, 1974.

M. C. Boskovic, T. B. Sekara, B. Lutovac, M. Dakovic, P. D. Mandic, M. P. Lazarevic, “Analysis of electrical circuits including fractional order elements,” in Proceedings of the 6th Mediterranean Conference on Embedded Computing, MECO, Bar, Montenegro, June 11-15, 2017, pp. 1–6.

M. C. Boskovic, M. R. Rapaic, T. B. Sekara, P. D. Mandic, M. P. Lazarevic, B. Cvetkovic, B. Lutovac, M. Dakovic, “On the rational representation of fractional order lead compensator using Padé approximation,” in Proceedings of the 7th Mediterranean Conference on Embedded Computing, MECO, Budva, Montenegro, June 10-14, 2018, pp. 1–4.

S. M. Cveticanin, D. Zorica, M. R. Rapaic, “Generalized time-fractional telegrapher's equation in transmission line modeling,” Nonlinear Dyn, 2017., DOI 10.1007/sI1071-016-3322-z

M. P. Lazarevic, P. D. Mandic, B. Cvetkovic, T. B. Sekara, B. Lutovac, “Some electromechanical systems and analogies of mem-systems integer and fractional order,” in Proceedings of the 5th Mediterranean Conference on Embedded Computing, MECO, Bar, Montenegro, June 12-16, 2016, pp. 230–233.

M. C. Boskovic, T. B. Sekara, M. R. Rapaic, B. Lutovac, M. Dakovic, V. Govedarica, “Novel band-pass and notch filter with dynamic damping of fractional order,” in Proceedings of the 25th Telecommunication Forum, TELFOR, Belgrade, Serbia, November 21-22, 2017, pp. 1–4.

D. Jurisic, E. Emanovic, B. Lutovac, G. S. Moschytz, “Noise analysis of fractional-order two-integrator CCII low-pass filter using Pspice,” in Proceedings of the 7th Mediterranean Conference on Embedded Computing, MECO, Budva, Montenegro, June 10-14, 2018, pp. 1–5.

G. Tsirimokou, C. Psychalinos, and A. Elwakil, Design of CMOS analog integrated fractional-order circuits: applications in medicine and biology. Springer, 2017.

M. E. Van Valkenburg, Analog Filter Design. CBS College Publishing, New York, U.S., 1982.

T. J. Freeborn, B. Maundy, and A. Elwakil, “Fractional-step Tow-Thomas biquad filters,” Nonlinear Theory and Its Applications, NOLTA, IEICE, vol. 3, pp. 357–374, July 2012.

T. J. Freeborn, B. Maundy, and A. S. Elwakil, “Approximated fractional order Chebyshev lowpass filters,” Mathematical Problems in Engineering, vol. 2015, Article ID 832468, 2015.

A. Oustaloup, F. Levron, B. Mathieu and F. M. Nanot, “Frequency-band complex noninteger differentiator: characterization and synthesis,” IEEE Trans Circ Syst– I: Fundam Theory App 2000; vol. 47, no. 1, pp. 25–39.

B. T. Krishna, “Studies on fractional-order differentiators and integrators: a survey,” Signal Process 2011, vol. 91, no. 3, pp. 386–426.

G. Tsirimokou, “A systematic procedure for deriving RC networks of fractional-order elements emulators using MATLAB,” AEU-International Journal of Electronics and Communications, vol. 78, pp. 7–14, August 2017.